CIVE202- Civil Engineering Analysis II – Project 5 Written Technical Report

Spring 2026

Jacob Oetken, Michael McCarthy, Tyler Thiede, Bryce Ripley

May 8, 2026

Table of Contents

Technical Executive Summary	2

Non-Technical Executive Summary	2

# Technical Executive Summary

This project created a fully automated Python notebook that executes tensile yielding design checks for W-Shaped steel tension members. All design checks follow the AISC 14th Edition Specifications and Example D.1. The program incorporated the AISC W shape section properties along with ASTM material yield strengths, validations of user inputs, computations of LRFD and ASD loads, and finally evaluated the members capacity using the set of equations shown in example D.1. The workflow calculates the nominal tensile strength, available LRFD and ASD strengths comparatively, and the percent utilization for a user designed scenario. Following this, three predefined scenarios are run and compared to the user's input design scenario.

The automated Python notebook produces a complete results table along with two utilization percentage plots that clearly demonstrate how each W shape tension member performs under a specified loading condition. All four scenarios evaluated passed both the LRFD and ASD checks. Scenario 2, being the W10X30, A36 steel, was the most critical case within our evaluation, reaching upwards of 80% utilization, as this scenario used the smallest cross-sectional area and had a lower yield strength. The user input of W12X50, A992 steel showed to have a middle ground utilization of 58%, demonstrating enough capacity for the load with a good margin of error included. Scenarios 1 and 3 provided the most conservative results due to the members selected having the largest cross-sectional area. Overall, the Python notebook created a reproducible, transparent, and proof-of-concept method in order to evaluate W shape members' tensile yielding capacities and gives clear visuals on how different member selections affect overall efficiency.

# Non-Technical Executive Summary

This project created a simple, proof-of-concept program that is intended to help engineers quickly and efficiently check whether a steel beam is strong enough to carry the load that it will experience in a structure. The tool uses information and equations from steel tables and manuals and directly applies them to a program that automatically carries out a calculation, eliminating the risk of human error. The program allows users to enter a specific steel shape, the material type, and a loading condition, then uses the information to calculate and evaluate the viability of that member for use in a specific building application.

To demonstrate the use of this program, we tested both a user-defined scenario and three pre-defined scenarios. All four of the scenarios passed the loading tests, but scenario 2 proved to be the most critical scenario as it was very close to its maximum capacity. Scenario 2 was the middle ground, and scenarios 1 and 3 showed the most efficiency in terms of capacity to load. The Python program also created a results table along with two bar charts showing how efficient each scenario is when it comes to loading capacity. Overall, the workflow provides a fast, accurate, and easily repeatable way to check design decisions and validate that selected steel members in construction applications are selected properly to ensure public safety.