# Methods

### Translate AISC Tensile Yielding Logic into Python

The AISC tensile yielding procedure for W-shaped tension members was implemented in Python using gross-section strength equations from AISC Chapter D. The program computes required tensile loads using standard LRFD and ASD load combinations and evaluates member capacity based on gross-section yielding. The nominal strength is calculated using the yield stress and gross cross-sectional area of the selected section, and design strengths are determined using AISC resistance and safety factors.

### Tensile Yielding of the Gross Section Area

The first calculation to be performed is for testing the tensile yielding of the gross section of the w-shaped tension member. This will require the inputs from the user for their values for Dead load and live load, and ASTM Identifier of the material for this specific scenario. The gross area will reference the input for member the AISC manual label and pull that value from the csv. Then to calculate the Load and Resistance Factor Design (LRFD) a Percent Utilization will be defined and a function as the sum of the Dead load multiplied by 1.2 and the Live load multiplied by 1.6.

The Allowable Strength Design will be defined as the function calculating the sum of the Live Load and Dead Load. The nominal yielding strength will be defined as a function multiplying the material property by the gross area of the w-shaped tension member. The LRFD Design Strength will be defined as a function multiplying the Nominal Yielding strength by 0.9 and saved as a variable for later calculations.

### Implement Sequential User Inputs

The Python environment will prompt the user for values as follows: W-Shape AISC Designation, Material ASTM Designation (Choices), Dead Load (kips), and Live Load (kips). In the case of inputting a designation for W-Shape, the program will search through the AISC Shapes Database V160-2 to find the desired W-Shape. For material choice, the user will be prompted with choices pertaining to table 2-5 from the AISC Steel Construction Manual 14ed.

### W-shaped tension member Calculation

The tensile yielding calculations were performed using equations from the AISC Specification for Structural Steel Buildings (ANSI/AISC 360), specifically Chapter D, Section D2 for gross-section yielding. Required tensile loads were computed using standard load combinations from ASCE 7. Geometric properties for W-shaped members were obtained from the AISC shapes database, while material properties were assigned based on ASTM steel grades consistent with AISC specifications. See equation list below:

Required Tensile Load (LRFD)

Pu=1.2D+1.6L

Required Tensile Load (ASD)

Pa=D+L

Nominal Tensile Yielding Strength

Pn=FyAg

LRFD Design Strength

ϕPn=0.90Pn

ASD Allowable Strength

Pallow=Pn1.67

LRFD Capacity Check

ϕPn​≥Pu​

ASD Capacity Check

Pn​​Ω≥Pa​

Percent Utilization (LRFD)

ULRFD=ϕPuPn100

Percent Utilization (ASD)

UASD=PaPn/Ω100

| Variable | Meaning | Units |
| --- | --- | --- |
| D | Dead Load | kips |
| L | Live Load | kips |
| Pu | Factored Tensile Load (LRFD) | kips |
| Pa | Service Tensile Load (ASD) | kips |
| Fy | Yield Stress | ksi |
| Ag | Gross area | in^2 |
| Pn | Nominal Tensile Strength | kips |
| ϕ | LRFD Resistance Factor (Yielding) | 0.90 |
| Ω | ASD Safety Factor (Yielding) | 1.67 |
| ϕPn | LRFD Available Strength | kips |
| U | ASD Allowable Strength | kips |

### Generate W-shaped tension member Design Output

Inputs:

- D, L, Fy (via Material), Ag (via W-shape)

Outputs:

- Pu, Pa

- ϕPn, Pn/Ω

- utilization

- pass/fail

### Prepare and Evaluate Three W-shaped member scenarios and graph the results

Three scenarios will be run through the program. Each scenario’s output will be printed in a formatted table as well as in a bar chart to help visualize which designation combinations will meet the requirements under the inputted dead and live tensile loads. In the printed table, several data values pertaining to the inputted beam designation and material will be printed. These values include the required tensile strength for LFRD and ASD, the nominal tensile yielding strength for LFRD and ASD, the LRFD design strength, the ASD allowable strength, the LRFD capacity check, and the percent utilization for LRFD and ASD. Three more tables will also be printed for our 3 scenarios.

Two bar charts will then be created with beam designations on the x-axis and percent utilization on the y-axis. One bar chart will include the LRFD percent utilization while the other bar chart will use the ASD. This graph will help show the capabilities of the 3 scenarios and the inputted scenario under the given tensile load to see which scenarios will fail and which are over-engineered to help prevent fracture and save costs.